SSC JE CSE – Computer Networks
Day 1 | Set 1 – 50 Questions

Question Bank with Answers & Short Explanations

Exam reference: 200 questions / 120 minutes ≈ 36 seconds per question. Recommended practice target for this set: 30 minutes initially, then improve speed.

Q1. Which layer of the OSI model is responsible for end-to-end delivery of data?

A. Network

B. Transport

C. Session

D. Data Link

Answer: B

Explanation: Transport layer provides end-to-end delivery.

Q2. Which OSI layer is responsible for logical addressing?

A. Physical

B. Data Link

C. Network

D. Transport

Answer: C

Explanation: The Network layer handles logical addressing such as IP addresses.

Q3. Which layer of the OSI model is responsible for framing?

A. Physical

B. Data Link

C. Network

D. Transport

Answer: B

Explanation: The Data Link layer organizes bits into frames.

Q4. Which of the following protocols operates at the Transport layer?

A. IP

B. ARP

C. TCP

D. Ethernet

Answer: C

Explanation: TCP is a Transport-layer protocol.

Q5. The OSI model consists of how many layers?

A. 5

B. 6

C. 7

D. 8

Answer: C

Explanation: The OSI reference model has seven layers.

Q6. Which layer is responsible for establishing, managing and terminating sessions between applications?

A. Transport

B. Session

C. Presentation

D. Application

Answer: B

Explanation: The Session layer manages communication sessions.

Q7. Encryption, compression and translation of data are primarily associated with which OSI layer?

A. Session

B. Transport

C. Presentation

D. Application

Answer: C

Explanation: The Presentation layer handles translation, compression and encryption-related representation functions.

Q8. Which of the following is NOT a layer of the TCP/IP model?

A. Internet

B. Transport

C. Session

D. Application

Answer: C

Explanation: Session is not a separate layer in the commonly used TCP/IP model.

Q9. The physical address of a network interface is commonly known as:

A. IP address

B. MAC address

C. Port number

D. Network address

Answer: B

Explanation: A MAC address identifies a network interface at the Data Link layer.

Q10. A MAC address is normally:

A. 16 bits

B. 32 bits

C. 48 bits

D. 64 bits

Answer: C

Explanation: A standard Ethernet MAC address is 48 bits.

Q11. Which protocol is used to determine the MAC address corresponding to an IPv4 address?

A. RARP

B. ARP

C. DHCP

D. DNS

Answer: B

Explanation: ARP resolves an IPv4 address to a MAC address on a local network.

Q12. Ethernet primarily operates at which OSI layers?

A. Physical and Data Link

B. Network and Transport

C. Transport and Session

D. Session and Presentation

Answer: A

Explanation: Ethernet standards cover Physical and Data Link layer functions.

Q13. Which device generally operates at the Data Link layer?

A. Hub

B. Switch

C. Repeater

D. Router

Answer: B

Explanation: A traditional Layer-2 switch forwards frames using MAC addresses.

Q14. CSMA/CD was traditionally associated with:

A. Ethernet

B. Token Ring

C. Bluetooth

D. ATM

Answer: A

Explanation: CSMA/CD was used by traditional shared-medium Ethernet.

Q15. What is the primary purpose of a switch?

A. Assign IP addresses

B. Forward frames based on MAC addresses

C. Translate domain names

D. Route packets between networks

Answer: B

Explanation: A Layer-2 switch forwards frames according to its MAC address table.

Q16. An IPv4 address contains:

A. 16 bits

B. 32 bits

C. 48 bits

D. 64 bits

Answer: B

Explanation: IPv4 addresses are 32 bits long.

Q17. Which of the following is a private IPv4 address?

A. 8.8.8.8

B. 172.20.10.5

C. 1.1.1.1

D. 202.10.10.1

Answer: B

Explanation: 172.20.10.5 lies within the private 172.16.0.0/12 range.

Q18. Which range belongs to the private Class A address space?

A. 10.0.0.0 – 10.255.255.255

B. 127.0.0.0 – 127.255.255.255

C. 172.0.0.0 – 172.255.255.255

D. 192.0.0.0 – 192.255.255.255

Answer: A

Explanation: 10.0.0.0/8 is the private IPv4 range traditionally associated with Class A.

Q19. The subnet mask 255.255.255.0 corresponds to:

A. /8

B. /16

C. /24

D. /32

Answer: C

Explanation: 255.255.255.0 has 24 leading one bits, so it is /24.

Q20. How many usable host addresses are available in a /24 IPv4 subnet?

A. 256

B. 255

C. 254

D. 252

Answer: C

Explanation: A /24 has 256 addresses; normally network and broadcast are reserved, leaving 254 usable hosts.

Q21. A network uses the subnet mask 255.255.255.192. How many usable hosts can each subnet support?

A. 30

B. 62

C. 64

D. 126

Answer: B

Explanation: 255.255.255.192 is /26: 64 total addresses, 62 normally usable hosts.

Q22. Which address represents the loopback address in IPv4?

A. 0.0.0.0

B. 127.0.0.1

C. 192.168.1.1

D. 255.255.255.255

Answer: B

Explanation: 127.0.0.1 is the commonly used IPv4 loopback address.

Q23. The address 255.255.255.255 is commonly used as:

A. Loopback address

B. Limited broadcast address

C. Private address

D. Multicast address

Answer: B

Explanation: 255.255.255.255 is the IPv4 limited broadcast address.

Q24. How many bits are used for an IPv6 address?

A. 32

B. 64

C. 128

D. 256

Answer: C

Explanation: IPv6 addresses are 128 bits long.

Q25. Which device is primarily responsible for routing packets between different networks?

A. Hub

B. Switch

C. Router

D. Repeater

Answer: C

Explanation: Routers forward packets between different IP networks.

Q26. Which routing protocol is a distance-vector routing protocol?

A. RIP

B. OSPF

C. BGP

D. IS-IS

Answer: A

Explanation: RIP is a classic distance-vector routing protocol.

Q27. RIP uses which metric to determine the best route?

A. Delay

B. Bandwidth

C. Hop count

D. Cost

Answer: C

Explanation: RIP selects routes using hop count.

Q28. What is the maximum hop count supported by RIP before a destination is considered unreachable?

A. 15

B. 16

C. 31

D. 32

Answer: A

Explanation: A route with 16 hops is considered unreachable; 15 is the maximum usable hop count.

Q29. OSPF is based primarily on which routing approach?

A. Distance vector

B. Link state

C. Path vector

D. Flooding only

Answer: B

Explanation: OSPF is a link-state routing protocol.

Q30. BGP is primarily used for:

A. Routing within a LAN

B. Routing between autonomous systems

C. Resolving IP addresses

D. Assigning MAC addresses

Answer: B

Explanation: BGP is the principal inter-domain routing protocol used between autonomous systems.

Q31. Which protocol is connection-oriented?

A. UDP

B. IP

C. TCP

D. ICMP

Answer: C

Explanation: TCP establishes a connection before data transfer.

Q32. Which protocol generally provides faster transmission with lower overhead?

A. TCP

B. UDP

C. FTP

D. HTTP

Answer: B

Explanation: UDP has lower overhead because it does not provide TCP's connection and reliability mechanisms.

Q33. TCP establishes a connection using:

A. Two-way handshake

B. Three-way handshake

C. Four-way handshake

D. Five-way handshake

Answer: B

Explanation: TCP connection establishment uses a three-way handshake.

Q34. The correct sequence for TCP connection establishment is:

A. ACK → SYN → SYN-ACK

B. SYN → ACK → SYN-ACK

C. SYN → SYN-ACK → ACK

D. SYN-ACK → SYN → ACK

Answer: C

Explanation: The client sends SYN, server replies SYN-ACK, and client sends ACK.

Q35. Which TCP mechanism is primarily responsible for controlling the amount of unacknowledged data in transit?

A. Flow control

B. DNS

C. ARP

D. Fragmentation

Answer: A

Explanation: TCP flow control uses the receiver's advertised window to regulate outstanding data.

Q36. TCP provides:

A. Unreliable connectionless delivery

B. Reliable ordered byte-stream delivery

C. Only broadcast delivery

D. Physical-layer delivery

Answer: B

Explanation: TCP provides reliable, ordered, full-duplex byte-stream service.

Q37. UDP is:

A. Connection-oriented

B. Connectionless

C. Circuit-switched

D. Reliable by default

Answer: B

Explanation: UDP is a connectionless transport protocol.

Q38. Which protocol is primarily used for translating domain names into IP addresses?

A. DHCP

B. DNS

C. FTP

D. SMTP

Answer: B

Explanation: DNS maps domain names to IP addresses and other DNS records.

Q39. Which protocol is used for transferring files between computers?

A. FTP

B. SMTP

C. SNMP

D. ARP

Answer: A

Explanation: FTP is the File Transfer Protocol.

Q40. SMTP is primarily used for:

A. Receiving web pages

B. Sending email

C. Assigning IP addresses

D. File compression

Answer: B

Explanation: SMTP is primarily used to send and relay email.

Q41. Which protocol is commonly used to automatically assign IP configuration to a host?

A. DNS

B. DHCP

C. FTP

D. ICMP

Answer: B

Explanation: DHCP can automatically provide IP address, subnet mask, gateway and DNS configuration.

Q42. HTTP normally operates at which OSI layer?

A. Transport

B. Network

C. Application

D. Data Link

Answer: C

Explanation: HTTP is an Application-layer protocol.

Q43. The default port number associated with HTTP is:

A. 20

B. 21

C. 80

D. 443

Answer: C

Explanation: HTTP commonly uses TCP port 80.

Q44. The default port number associated with HTTPS is:

A. 25

B. 53

C. 110

D. 443

Answer: D

Explanation: HTTPS commonly uses TCP port 443.

Q45. Which device regenerates an incoming signal and sends it onward?

A. Router

B. Repeater

C. Switch

D. Gateway

Answer: B

Explanation: A repeater regenerates and retransmits signals at the Physical layer.

Q46. Which topology has a central connecting device?

A. Bus

B. Ring

C. Star

D. Mesh

Answer: C

Explanation: In a star topology, nodes connect through a central device.

Q47. Which protocol is primarily used to report network-layer errors and diagnostic information?

A. TCP

B. UDP

C. ICMP

D. ARP

Answer: C

Explanation: ICMP carries network-layer control, error and diagnostic messages.

Q48. Which of the following provides encryption for web communication?

A. HTTP

B. HTTPS

C. FTP

D. Telnet

Answer: B

Explanation: HTTPS uses HTTP over TLS to provide encrypted communication.

Q49. Which protocol is generally considered insecure because it transmits data, including credentials, without encryption?

A. SSH

B. HTTPS

C. Telnet

D. SFTP

Answer: C

Explanation: Traditional Telnet sends communication in plaintext; SSH is the secure alternative.

Q50. Which statement is correct?

A. Router forwards frames using MAC addresses only

B. Switch primarily forwards packets using IP addresses

C. Router operates primarily at the Network layer

D. Hub operates at the Transport layer

Answer: C

Explanation: A traditional router primarily operates at the Network layer and forwards packets using IP routing information.

Quick Revision – High-Yield CN Facts

• OSI has 7 layers: Physical, Data Link, Network, Transport, Session, Presentation, Application.

• IPv4 = 32 bits; IPv6 = 128 bits; standard MAC address = 48 bits.

• ARP maps IPv4 address to MAC address on a local network.

• TCP is connection-oriented and uses a three-way handshake: SYN → SYN-ACK → ACK.

• UDP is connectionless and has lower overhead than TCP.

• RIP uses hop count; 16 hops is considered unreachable, so 15 is the maximum usable hop count.

• OSPF is a link-state routing protocol; BGP is used for inter-autonomous-system routing.

• HTTP commonly uses port 80; HTTPS commonly uses port 443.

• DNS resolves domain names; DHCP provides automatic network configuration.

• Router: primarily Network layer. Traditional switch: Data Link layer. Repeater/hub: Physical layer.