SSC JE CSE – Computer Networks
DAY 2 | SET 2 – 50 QUESTIONS

Answers + Short Explanations

Practice target: 30 minutes. Exam benchmark: approximately 36 seconds per question.

Q1. A network has subnet mask 255.255.255.224. How many usable host addresses are available per subnet?

A. 14

B. 30

C. 32

D. 62

Answer: B

Explanation: /27 gives 32 total addresses and 30 usable hosts.

Q2. What is the CIDR notation of 255.255.255.240?

A. /26

B. /27

C. /28

D. /29

Answer: C

Explanation: 255.255.255.240 has 28 network bits.

Q3. A /26 network contains how many total IPv4 addresses?

A. 32

B. 64

C. 128

D. 256

Answer: B

Explanation: 2^(32−26) = 64.

Q4. How many usable host addresses are available in a /27 subnet?

A. 30

B. 31

C. 32

D. 62

Answer: A

Explanation: 32 total addresses minus network and broadcast = 30.

Q5. Which subnet mask provides 14 usable host addresses?

A. 255.255.255.224

B. 255.255.255.240

C. 255.255.255.248

D. 255.255.255.252

Answer: B

Explanation: /28 gives 16 total and 14 usable addresses.

Q6. 192.168.10.65/26 belongs to which network?

A. 192.168.10.0

B. 192.168.10.32

C. 192.168.10.64

D. 192.168.10.128

Answer: C

Explanation: A /26 has blocks of 64; .65 is in the .64 network.

Q7. For 192.168.1.64/26, which is the broadcast address?

A. 192.168.1.63

B. 192.168.1.64

C. 192.168.1.126

D. 192.168.1.127

Answer: D

Explanation: The .64–.127 block has .127 as broadcast.

Q8. Which is a valid host in 192.168.1.128/27?

A. 192.168.1.128

B. 192.168.1.159

C. 192.168.1.160

D. 192.168.1.150

Answer: D

Explanation: .128 is network and .159 is broadcast; .150 is a valid host.

Q9. How many /24 networks can be created from a /16 network?

A. 8

B. 16

C. 256

D. 512

Answer: C

Explanation: 8 bits are borrowed: 2^8 = 256.

Q10. A company needs at least 50 usable hosts per subnet. Smallest suitable subnet?

A. /25

B. /26

C. /27

D. /28

Answer: B

Explanation: /26 provides 62 usable hosts.

Q11. Which IPv4 field prevents packets from circulating indefinitely?

A. Identification

B. TTL

C. Protocol

D. Header checksum

Answer: B

Explanation: TTL is decremented by routers.

Q12. If IPv4 TTL becomes zero at a router, what happens?

A. Forwarded

B. Discarded

C. Sent to DNS

D. Retransmitted by TCP

Answer: B

Explanation: The packet is discarded when TTL expires.

Q13. Which IPv4 field identifies the upper-layer protocol?

A. TTL

B. Protocol

C. Identification

D. IHL

Answer: B

Explanation: The Protocol field identifies TCP, UDP, ICMP, etc.

Q14. Which protocol provides reliable delivery over IP?

A. UDP

B. TCP

C. ICMP

D. ARP

Answer: B

Explanation: TCP provides reliable transport service.

Q15. TCP sequence numbers primarily:

A. Identify destination IP

B. Order bytes and support reliability

C. Identify MAC

D. Select route

Answer: B

Explanation: They support ordering and retransmission.

Q16. Which TCP flag requests connection establishment?

A. FIN

B. ACK

C. SYN

D. RST

Answer: C

Explanation: SYN starts TCP connection establishment.

Q17. Which TCP flag is used for graceful termination?

A. SYN

B. FIN

C. ACK

D. PSH

Answer: B

Explanation: FIN indicates the sender has finished sending.

Q18. Primary purpose of TCP ACK?

A. Select route

B. Confirm received data

C. Encrypt packet

D. Assign IP

Answer: B

Explanation: ACK confirms receipt of data.

Q19. TCP congestion control primarily attempts to:

A. Increase loss

B. Prevent network overload

C. Replace DNS

D. Assign MAC

Answer: B

Explanation: It regulates sending rate to control congestion.

Q20. Which is associated with TCP congestion control?

A. Slow start

B. ARP

C. DNS

D. DHCP

Answer: A

Explanation: Slow start is a TCP congestion-control mechanism.

Q21. TCP flow control primarily prevents:

A. Sender overwhelming receiver

B. Router changing routes

C. DNS failure

D. IP duplication

Answer: A

Explanation: The receive window controls how much data can be outstanding.

Q22. Increasing sliding-window size generally allows:

A. Fewer outstanding packets

B. More outstanding packets

C. No packets

D. One packet only

Answer: B

Explanation: A larger window permits more unacknowledged data.

Q23. In Go-Back-N ARQ, if a packet is lost, sender generally:

A. Retransmits only it

B. Retransmits it and subsequent unacknowledged packets

C. Stops

D. Changes IP

Answer: B

Explanation: Go-Back-N retransmits from the missing packet onward.

Q24. Selective Repeat differs from Go-Back-N because:

A. Cannot retransmit

B. Retransmits only missing/damaged packets

C. No ACKs

D. No sequence numbers

Answer: B

Explanation: Selective Repeat retransmits selected packets.

Q25. Which allows multiple frames before waiting for individual ACKs?

A. Stop-and-wait

B. Sliding window

C. Circuit switching

D. Polling

Answer: B

Explanation: Sliding window permits multiple outstanding frames.

Q26. Which is commonly used for error detection?

A. CRC

B. DNS

C. DHCP

D. FTP

Answer: A

Explanation: CRC is a common error-detection method.

Q27. CRC stands for:

A. Cyclic Redundancy Check

B. Computer Routing Control

C. Central Redundancy Code

D. Cyclic Routing Calculation

Answer: A

Explanation: CRC = Cyclic Redundancy Check.

Q28. In CRC, the generator polynomial is used for:

A. Encryption

B. Polynomial division

C. IP addressing

D. Routing

Answer: B

Explanation: CRC uses modulo-2 polynomial division.

Q29. Which is generally more powerful for detecting burst errors?

A. CRC

B. Simple parity

C. ASCII

D. DNS

Answer: A

Explanation: CRC is effective for burst-error detection.

Q30. A parity bit is primarily used for:

A. Error detection

B. Routing

C. Encryption

D. Compression

Answer: A

Explanation: Parity provides simple error detection.

Q31. Which protocol resolves IPv4 address to MAC address?

A. DNS

B. ARP

C. DHCP

D. ICMP

Answer: B

Explanation: ARP performs IPv4-to-MAC resolution.

Q32. A switch learns MAC-to-port mapping using:

A. Source MAC of incoming frames

B. Destination IP only

C. DNS

D. TCP port

Answer: A

Explanation: Switches learn from source MAC addresses.

Q33. Unknown destination MAC at a typical Layer-2 switch causes:

A. Immediate drop

B. Flooding to other appropriate ports

C. DNS query

D. Conversion to IP

Answer: B

Explanation: Unknown unicast frames are normally flooded within the VLAN.

Q34. A hub generally:

A. Maintains MAC table

B. Forwards traffic to other ports

C. Performs IP routing

D. Performs congestion control

Answer: B

Explanation: A hub repeats the signal to its other ports.

Q35. Traditional Ethernet CSMA/CD uses:

A. DNS

B. Collision detection

C. TCP checksum

D. ARP

Answer: B

Explanation: CSMA/CD means Carrier Sense Multiple Access with Collision Detection.

Q36. Which Ethernet device separates collision domains?

A. Hub

B. Switch

C. Repeater

D. Coaxial cable

Answer: B

Explanation: Each switch port normally forms a separate collision domain.

Q37. Which routing protocol uses Shortest Path First?

A. RIP

B. OSPF

C. BGP

D. ARP

Answer: B

Explanation: OSPF uses SPF.

Q38. Algorithm commonly associated with OSPF?

A. Bellman-Ford

B. Dijkstra SPF

C. RSA

D. Kruskal

Answer: B

Explanation: OSPF uses Dijkstra's shortest-path algorithm.

Q39. Which is a path-vector routing protocol?

A. RIP

B. OSPF

C. BGP

D. ARP

Answer: C

Explanation: BGP is a path-vector protocol.

Q40. Which routing protocol uses hop count as primary metric?

A. OSPF

B. RIP

C. BGP

D. Ethernet

Answer: B

Explanation: RIP uses hop count.

Q41. Main purpose of a routing table?

A. Store MAC only

B. Determine packet forwarding

C. Store passwords

D. Store DNS names

Answer: B

Explanation: Routing tables determine next-hop/interface decisions.

Q42. Which routing approach automatically adjusts routes after topology changes?

A. Dynamic routing

B. Static routing

C. Manual forwarding

D. Physical addressing

Answer: A

Explanation: Dynamic routing protocols update routes as topology changes.

Q43. Which DNS record maps a domain name to IPv4?

A. MX

B. A

C. CNAME

D. PTR

Answer: B

Explanation: A records map names to IPv4 addresses.

Q44. Which DNS record contains mail-server information?

A. A

B. MX

C. PTR

D. AAAA

Answer: B

Explanation: MX records specify mail exchange servers.

Q45. Which protocol is used for secure remote login?

A. Telnet

B. SSH

C. FTP

D. HTTP

Answer: B

Explanation: SSH provides secure remote login.

Q46. Which protocol is commonly used for network management?

A. SNMP

B. SMTP

C. FTP

D. ARP

Answer: A

Explanation: SNMP is used for network management and monitoring.

Q47. Which switching technique receives the entire frame before forwarding?

A. Cut-through

B. Store-and-forward

C. Fragment-free

D. Circuit switching

Answer: B

Explanation: Store-and-forward receives and checks the complete frame.

Q48. Which statement is true about UDP?

A. Guarantees delivery

B. Guarantees ordering

C. No connection-establishment handshake

D. TCP-style congestion control

Answer: C

Explanation: UDP is connectionless and does not establish a TCP-style connection.

Q49. Which address identifies a process/service on a host?

A. MAC

B. IP

C. Port number

D. Subnet mask

Answer: C

Explanation: Transport-layer port numbers identify services/process endpoints.

Q50. Which protocol converts a website domain name into an IP address?

A. FTP

B. DNS

C. DHCP

D. ARP

Answer: B

Explanation: DNS resolves domain names to IP addresses.

DAY 2 – QUICK REVISION

• /24 = 256 total / 254 usable; /25 = 128 / 126; /26 = 64 / 62; /27 = 32 / 30; /28 = 16 / 14.

• TCP = reliable, ordered, connection-oriented. UDP = connectionless, lower overhead.

• TCP connection establishment: SYN → SYN-ACK → ACK.

• Go-Back-N retransmits from the missing frame onward; Selective Repeat retransmits selected missing/damaged frames.

• CRC is a polynomial-division based error-detection technique.

• RIP → distance vector → hop count; OSPF → link state → Dijkstra/SPF; BGP → path vector.

• DNS: A = IPv4, AAAA = IPv6, MX = mail server.

• SSH = secure remote login; SNMP = network management.