SSC JE CSE – COMPUTER NETWORKS
DAY 3 | SET 3 – 50 QUESTIONS

Questions + Options + Correct Answers

Practice target: 30 minutes. Exam benchmark: approximately 36 seconds per question.

Q1. An IPv4 address is 172.16.5.130/25. What is its network address?

A. 172.16.5.0

B. 172.16.5.64

C. 172.16.5.128

D. 172.16.5.255

Answer: C

Q2. For 192.168.10.0/28, which is the broadcast address?

A. .14

B. .15

C. .16

D. .31

Answer: B

Q3. How many subnets are created when a /24 network is divided into /27 networks?

A. 2

B. 4

C. 8

D. 16

Answer: C

Q4. What is the first usable host address of 192.168.20.64/27?

A. .64

B. .65

C. .66

D. .95

Answer: B

Q5. What is the last usable host address of 192.168.20.64/27?

A. .93

B. .94

C. .95

D. .96

Answer: B

Q6. Which subnet mask provides 62 usable host addresses?

A. /25

B. /26

C. /27

D. /28

Answer: B

Q7. An organization requires 1000 usable hosts in one IPv4 subnet. Which is the smallest suitable prefix?

A. /20

B. /21

C. /22

D. /23

Answer: C

Q8. How many usable hosts are available in a /22 network?

A. 512

B. 1022

C. 1024

D. 2046

Answer: B

Q9. Which is a valid IPv4 multicast address?

A. 127.0.0.1

B. 192.168.1.1

C. 224.0.0.5

D. 255.255.255.255

Answer: C

Q10. Which IPv4 address represents “this host” when the specific address is not yet known?

A. 0.0.0.0

B. 127.0.0.1

C. 255.255.255.255

D. 224.0.0.1

Answer: A

Q11. Which layer is responsible for detecting errors in frames?

A. Physical

B. Data Link

C. Network

D. Transport

Answer: B

Q12. Which of the following is a flow-control protocol?

A. Stop-and-Wait

B. DNS

C. ARP

D. ICMP

Answer: A

Q13. In Stop-and-Wait ARQ, the sender generally waits for:

A. Routing table

B. Acknowledgement

C. DNS response

D. MAC address

Answer: B

Q14. Which error-detection technique uses a divisor/generator polynomial?

A. Parity

B. Checksum

C. CRC

D. Hamming distance

Answer: C

Q15. A CRC generator has degree 4. How many CRC bits are appended?

A. 2

B. 3

C. 4

D. 5

Answer: C

Q16. Which technique can detect and correct single-bit errors?

A. CRC

B. Hamming code

C. Parity only

D. Checksum only

Answer: B

Q17. The minimum Hamming distance required to detect d errors is:

A. d

B. d + 1

C. 2d

D. 2d + 1

Answer: B

Q18. The minimum Hamming distance required to correct t errors is:

A. t + 1

B. 2t

C. 2t + 1

D. t − 1

Answer: C

Q19. Which sublayer of the Data Link layer is responsible for MAC addressing?

A. LLC

B. MAC

C. Network

D. Transport

Answer: B

Q20. What is the minimum Ethernet frame size traditionally used in standard Ethernet?

A. 32 bytes

B. 46 bytes

C. 64 bytes

D. 128 bytes

Answer: C

Q21. The maximum standard Ethernet frame size without VLAN tagging is commonly:

A. 512 bytes

B. 1024 bytes

C. 1518 bytes

D. 2048 bytes

Answer: C

Q22. Which address is used for Ethernet broadcast?

A. 00:00:00:00:00:00

B. FF:FF:FF:FF:FF:FF

C. 11:11:11:11:11:11

D. 01:00:00:00:00:00

Answer: B

Q23. Which device operates primarily at Layer 2?

A. Router

B. Switch

C. Repeater

D. Hub

Answer: B

Q24. A switch finds the destination MAC address in its table. What does it normally do?

A. Broadcast it

B. Forward through corresponding port

C. Drop it

D. Send to DNS

Answer: B

Q25. Which TCP mechanism controls sender transmission rate based on network congestion?

A. Flow control

B. Congestion control

C. Address resolution

D. Fragmentation

Answer: B

Q26. Which TCP congestion-control phase initially increases the congestion window rapidly?

A. Slow start

B. Fast termination

C. DNS resolution

D. ARP

Answer: A

Q27. TCP congestion window is commonly represented by:

A. rwnd

B. cwnd

C. ttl

D. mtu

Answer: B

Q28. Which TCP mechanism is used when duplicate ACKs indicate possible packet loss?

A. Fast retransmit

B. DNS

C. ARP

D. DHCP

Answer: A

Q29. Which protocol is more suitable for real-time voice/video where low delay is important?

A. TCP

B. UDP

C. FTP

D. SMTP

Answer: B

Q30. Which of the following does UDP NOT provide by itself?

A. Port numbers

B. Checksum

C. Guaranteed delivery

D. Connectionless communication

Answer: C

Q31. Which algorithm is traditionally associated with distance-vector routing?

A. Dijkstra

B. Bellman-Ford

C. RSA

D. Prim’s

Answer: B

Q32. Which algorithm is associated with link-state routing?

A. Bellman-Ford

B. Dijkstra

C. Floyd-Warshall only

D. RSA

Answer: B

Q33. Which routing protocol has a maximum usable hop count of 15?

A. OSPF

B. RIP

C. BGP

D. EIGRP

Answer: B

Q34. What happens when a router has no matching route and no default route?

A. Broadcasts packet

B. Normally discards packet

C. Sends to DNS

D. Converts to ARP

Answer: B

Q35. Which protocol exchanges routing information between autonomous systems?

A. RIP

B. OSPF

C. BGP

D. ARP

Answer: C

Q36. A default route is commonly represented as:

A. 127.0.0.0/8

B. 255.255.255.255/32

C. 0.0.0.0/0

D. 224.0.0.0/4

Answer: C

Q37. Which protocol reports errors such as Destination Unreachable?

A. TCP

B. UDP

C. ICMP

D. ARP

Answer: C

Q38. Which technique allows a private IP network to communicate with the public Internet using address translation?

A. NAT

B. ARP

C. DNS

D. FTP

Answer: A

Q39. Which protocol commonly uses port 53?

A. HTTP

B. DNS

C. FTP

D. SMTP

Answer: B

Q40. Which protocol commonly uses port 25?

A. SMTP

B. DNS

C. HTTP

D. SSH

Answer: A

Q41. Which protocol commonly uses port 22?

A. FTP

B. SSH

C. DNS

D. Telnet

Answer: B

Q42. Which protocol is commonly associated with port 23?

A. Telnet

B. SSH

C. HTTPS

D. SMTP

Answer: A

Q43. Which protocol is commonly used to retrieve email from a mail server and download messages?

A. POP3

B. SMTP

C. DNS

D. ARP

Answer: A

Q44. Which protocol keeps email synchronized on the server across multiple devices?

A. POP3

B. IMAP

C. FTP

D. DHCP

Answer: B

Q45. Which OSI layer provides logical addressing and routing?

A. Data Link

B. Network

C. Transport

D. Session

Answer: B

Q46. Which OSI layer provides process-to-process communication?

A. Physical

B. Data Link

C. Transport

D. Network

Answer: C

Q47. Which is a connectionless network-layer protocol?

A. IP

B. TCP

C. FTP

D. SMTP

Answer: A

Q48. Which device can connect networks using different protocols and perform protocol conversion?

A. Hub

B. Repeater

C. Gateway

D. Bridge

Answer: C

Q49. What is the correct order of encapsulation moving down the TCP/IP stack?

A. Data → Packet → Segment → Frame

B. Data → Segment → Packet → Frame

C. Frame → Packet → Segment → Data

D. Segment → Data → Frame → Packet

Answer: B

Q50. Which statement is correct?

A. TCP is connectionless and UDP connection-oriented

B. UDP guarantees delivery but TCP does not

C. TCP provides reliable ordered delivery, while UDP does not guarantee delivery

D. Both guarantee delivery

Answer: C

DAY 3 – QUICK REVISION

• /24=254 usable; /25=126; /26=62; /27=30; /28=14.

• Detect d errors → minimum Hamming distance d+1; correct t errors → 2t+1.

• CRC uses generator-polynomial division.

• Ethernet broadcast MAC = FF:FF:FF:FF:FF:FF.

• RIP → distance vector → Bellman-Ford → hop count.

• OSPF → link state → Dijkstra/SPF.

• BGP → path vector → inter-AS routing.

• Default IPv4 route = 0.0.0.0/0.

• DNS 53, SMTP 25, SSH 22, Telnet 23, POP3 110, IMAP 143.

• TCP is reliable/ordered/connection-oriented; UDP is connectionless with no delivery guarantee.